Chugai Technos Corporation

February 13, 2017

The spread of methods for determination of emission matters in flue gas  (Draft)

It’s necessary to preserve the air environment in order to protect human health and preserve living environment.

We must grasp accurate air pollutant concentration exhausted from stationary sources (factories, plants, etc.) and determinate emission standards.

Vietnamese government estimated TCVN by quoting from international standards such as United States Environmental Protection Agency or Internaional Organization for Standardization (ISO), and controls measuring organizations as regulations to mearsure harmful substances (dust, sulfur oxides, etc.) in flue gas exhausted from stationary sourses at present. However, the measuring equipment and methods are judged by each measuring organization, so that it’s hard to say that the methods are standardized. Also some measuring equipment is large scale such as auto isokinetic sampling equipment and expensive and there are some issues as follow.

- The measurement of flue gas should be separated from air environment originally, but they are not specifically identified.

- Several kinds of equipment and measuring methods are described in EPA and ISO. However, when auto isokinetic sampling equipmen, for instance, is used for dust measurement, it is carried out at only typical point such as center of duct because sampling equipment consist on heavy sampling tube and glassware and operators manage to traverse sampling tube. (Dust sampling shall be carried out at all measuring points depending on shape and size of duct in international standards.)

- The performance to remove substances (such as moisture and dust) that give data influence is not clear.

- When continuous measuring equipment is used, it is necessary to calibrate and select the measuring range before used. However, it’s uncertain, and simple measuring equipment (Testo) is used most of the time.

- Measuring flow rate of flue gas is always required for the calculation of total emission. However, sometimes it’s not carried out.

- The validation of the measurement results is difficult because enough data is not written.

Japanese Industrial Standards (JIS), which is estimated by Japanese government as measuring meathods by refering from EPA method, is improved through the coopration of the Ministory of Economy, Trade and Industry, universities, power companies and plant constructor,etc. The improvement is continued still now.

Despite the dominance of capital strength, the policy of JIS is that if the mearusing organizations have constant techniques and measuring equipment, the same results should be obtained. JIS is not only simple, convenient and measured at stationary sources, but also widely used for performance testing on boilers, incinerators and pollution preventive facilities.

To disseminate the JIS method in Vietnam, this proposal is to fostor flue gas measuring operators by holding technical seminors at major cities regarding the outline of measuring method, practical work and the equipment.

Chugai Technos have world wide experiences and is able to provide the measuring equipment and consumable goods stably and conduct technical training.

1.Outline of Planning

(1) Host City

Hanoi and Ho-Chi-Minh

(2) Date

Planning in summer 2017

(3) Eligible Person

Private and governmental flue gas mearusing operators in Vietnam

(4) Objective Measuring Analysis Method for Dissemination

Flow rate of flue gas, dust concentration, sulfer oxides concentaration, nitrogen oxides concentaration and monitor within JIS

(5) Outline of Technical Seminor

① Provide and lecture texts regarding measuring

② Explain the mearsuing methods through practical work and using sample ducts

③ Take reservation for exhibitions and purchace orders of related equipment

2.Technical Seminor

(1) Cost

Chugai is responsible for all cost associated to the seminors except for transportation fees of the attendees.

Due to the matter of budget, the size of seminor and detailed contents will be consulted.

(2) Application of Attendees

CEM is responsible for organizing attendees’ application.