CHÚ DẪN:

1 Phần côn phân kỳ E

2 Cổ đo hình trụ, C

3 Phần côn hội tụ B

4 Đầu vào hình trụ A

5 Mặt phẳng nối

a 7° ≤  ≤ 15°

b Hướng dòng chảy

c Xem 5.4.7

I. Cấu trúc của hệ thống chuẩn gió

1. Thiết bị nắn dòng dạng tổ ong (Honeycomb Flow Straightener)

- Aluminum

- 4-inch deep hexagonal cells

- Removable for cleaning

- Turbulence-reducing Screens

• Two 20 x 20 (mesh) screens made of 0.009” (0.23mm) diameter stainless steel wire Contraction (bell mouth) Contour

- Proprietary design used on all AEROLAB wind tunnels

- 9.5:1 contraction ratio Static Pressure Ring

- Access to test section static pressure for non-intrusive q/V measurements

- Consists of four pressure ports – one on each wall just prior to test section entrance w/ manifold Test Section

- Hard anodized aluminum structure

- Two top-hinged Acrylic side windows and one top window

- Integrated yaw table w/ engraved scale and prepared for optional model positioning system Diffuser Design

• Increases wind tunnel efficiency and overall flow quality Motor
• High-efficiency 10 hp (7.5 KW) electric motor Fan/Propeller
• High-efficiency
• 9-blade
• Balanced
Noise Attenuator
• Reduces noise w/ 2 layers of acoustic insulation Speed Controller

- Solid state Variable Frequency Drive (VFD)

- Accepts analog or digital remote control signals

- Built-in display is selectable for Hz, RPM, % power and Amps

- Adaptable to 208-240VAC, 380-480VAC and 50/60Hz Excellent Flow Quality

• Less than 0.2% turbulence
Functionality
• Easy to use
Mobility
• Four high-quality steel casters with polyurethane wheels